Titanium Rimfire Suppressor — Cleaning & Care Instructions

Safety first: always ensure the firearm is unloaded and follow normal range and workshop safety procedures before removing or handling a suppressor. Wear eye and glove protection when cleaning.

Rimfire ammunition produces more fouling per round than centerfire, so a slightly tighter cleaning rhythm is prudent. Below is a clear, practical schedule and guidance to keep a rimfire titanium suppressor running reliably and looking good.

General Guidelines

- Visually inspect the suppressor regularly (every few range sessions) to confirm there is no excessive carbon buildup or residue that could affect performance.

- At every cleaning interval, remove the hub and end cap and check threads and mating surfaces for fouling or damage.

- Remove small internal parts (pistons, springs, sleeves) and clean them separately — these are typically stainless steel.

- Follow the cleaner manufacturer’s directions for soak times, rinsing, and drying.

- Although a 500-round schedule is reasonable, shorten intervals if you notice accelerated fouling or run rapid strings of fire.

Approved Cleaners (examples)

- Otis Suppressor Cleaner

- Simple Green® Extreme / Simple Green® XL (use formulas safe for metals)

- Mild solvent baths (non-ammonia, manufacturer-recommended)

Avoid ammonia-based cleaners, aggressive acids/alkalis, and household detergents that list aluminum corrosion on the label.

Brushes & Tools

- Use nylon brushes and non-metallic picks only — metal brushes can damage threads, coatings, and mating surfaces.

- Ultrasonic cleaners are acceptable for titanium rimfire suppressors; follow recommended cycle times and cleaner guidance.

- Avoid harsh abrasives, steel wool, or aggressive scrubbing that can scratch or pit the finish.

Suggested Cleaning Intervals & Procedures

Light cleaning — After ~200–300 rounds

- Quick inspection and light cleaning.

- Wipe visible carbon deposits and check thread faces for residue.

- Confirm coatings/finishes show no unusual discoloration or wear.

Routine cleaning — After ~500 rounds

- Perform a longer soak in an approved cleaner (follow manufacturer instructions).

- Remove hub and end cap; scrub accessible parts with a nylon brush.

- Inspect end cap for strikes and mating surfaces for fouling.

Deep clean — After ~1,000 rounds

- Perform an extended soak using an approved cleaner per directions.

- Use an ultrasonic cleaner if available (keep cycles moderate).

- Rinse, dry completely, and lightly lubricate or apply light anti-seize to external threads before reassembly.

Additional Notes & Best Practices

- Because rimfire fouling accumulates faster, adopt a conservative approach: if you shoot frequent short strings or use corrosive ammo (rare for modern rimfire but worth noting), clean sooner.

- Do not leave parts submerged longer than recommended by the cleaner’s instructions. Prolonged exposure can harm finishes or small components.

- Light anti-seize or a high-temperature grease on external threads helps prevent carbon lock — avoid getting grease in bore paths or gas ports.

- Store the suppressor dry and ventilated after cleaning.

- If you observe heavy baffle erosion, cracked parts, or thread damage, discontinue use and consult the manufacturer or a qualified gunsmith.