Summary of Work

Project Name: 	French Valley 48

Project Location: French Valley, CA

Client Name:

Date of Estimate:11/3/2024	Estimate Version: Rev.1

Prepared by: Subgrade Services LLC

Scope of Work

- Project Type: Subdivision

- Scope: Budget Cut Fill to grade

- Rough Grade Takeoff

Sources and Assumptions

• Documents Used: Tr 38034 RGRD-BGR2400083_FINAL; 3646CR Updated Geotechnical Report French Valley 48

• Topography Source: CAD-2394 GRD; 2394 TOPO.

• Assumptions: -Remedial soil removals up to 8 feet below existing grade. 3-foot PAD OX from Design/Existing.

1. Earthwork

- Cut Volume: 24,229 CY

- Remedial Cut Volume: 87,939 CY

- Additional Cut Volume:2148 CY

- Total Cut Volume:114,316 CY

- Total Fill Volume: 117,082 CY (131,132 CCY)

- Stripping and Grubbing: 5 ACRES

- Mass Haul: (as applicable) -16815 CY

2. Utilities

• Storm Drainage: 2881 LF

• Water Line: 1885 LF

• Sewer Line: 4221 LF

3. Pavement

• Subgrade Preparation: 125,197 SF

• Aggregate Base: NA

• Asphalt Concrete: NA

4. Other Items

• Retaining Walls: NA

• Curb and Gutter: NA

• Sidewalks: NA

Summary Notes and Recommendations

Job will be an import due to compaction and shrinking of the soil.

Soil Shink assumption -12% (volumes given are bank)

Spoils from trenches are not included on the report.

Disclaimer: This quantity takeoff estimate is based on the information provided as of the date of this document. Changes to project specifications, drawings, or site conditions may impact estimated quantities.

Prepared by: Subgrade Services LLC

Contact Information: