2017-2025 Sidewinders

These are base settings for the HP tune you are running.

The clutching was based off all Hurricane Performance tunes.

Tunes from other companies will affect weight.

Studded or non-studded tracks will affect weight.

Weight of the rider and machine will affect weight.

81-83 grams 240-250 tune.

84 grams 270 tune.

85-86 grams 300 tune.

Start by adding weight closest to tip first.

Secondary setting.

  - is a good starting point and most likely won’t be changed.

  - would make it run about 100 RPMs higher. 0-1 would lower RPMs about 100.

We recommend setting your clutch offset at 2.25” or 57.5mm.

Some machining to the inner post and aluminum spacer will be needed to get proper offset. Machine the post and spacer equal amounts. We have been finding that to be the best offset. Running it with more or less offset could result in blown belts.