SEVEN STATES POWER CORPORATION

REQUEST FOR PROPOSALS

MIDDLE MILE FIBER OPTIC NETWORK

ENGINEERING DESIGN

| #### Key Dates |  |
| --- | --- |
| #### RFP Release Date | #### 3/1/2021 |
| #### Non-Disclosure Agreement Due Date | #### 3/4/2021 |
| #### Bidders Conference Call | #### 3/8/2021 |
| #### Bidders Question Submission Deadline | #### 3/15/2021 |
| #### RFP Responses Due | #### 4/12/2021 |
| #### Target RFP Award Date | #### 6/1/2021 |

# ABSTRACT

Seven States Power Corporation (Seven States), in partnership with local power companies (LPCs) across the Tennessee Valley, is accepting proposals from qualified engineering firms to design a middle mile fiber optic backhaul network across the seven-states region of Tennessee and parts of Alabama, Georgia, Kentucky, Mississippi, North Carolina and Virginia.

The network will connect 153 local power companies (LPC) utilizing existing fiber when available, and building new fiber routes when not available.  The network will leverage existing LPC fiber infrastructure and build connections between systems to form a fully integrated fiber network across the seven-state region.

The purpose of the interconnected network is two-fold.  It will serve as a tool to improve operational efficiencies and reliability for LPCs.  Potential applications include enhanced emergency communications, data warehousing, load management services, aggregated billing systems, and shared access to centralized applications.  The interconnected network will also provide a path for LPCs to develop new revenue streams by offering broadband services to its customers by tapping into headend infrastructure built outside each LPCs service territory.  This will reduce the financial risk by minimizing capital investments required for LPCs entering the market.

The RFP seeks to identify qualified firms (Bidders) to provide in-house engineering resources to design the fully integrated fiber optic network.  The design will incorporate existing LPC fiber infrastructure merged with new fiber construction to complete the fiber network.  In addition to designing the fiber network, the successful bidder shall provide construction estimates for building new fiber routes and equipment estimates for interconnection and aggregation locations.

# ABOUT SEVEN STATES POWER CORPORATION

Seven States Power is an energy solutions company that serves 153 local power companies across seven states of the Tennessee Valley. Our members distribute TVA-generated electricity and provide a variety of essential services that build the economy and enhance life in their communities.

Our turnkey products and services include EV (electric vehicle) chargers, solar arrays, backup generation, combined heat and power, battery storage, fiber deployment, cybersecurity, economic analysis and distributed energy resource management systems.

Seven States Power, formed in 2007, is a non-profit membership corporation with the powers of a generation & transmission cooperative. In early 2020, Seven States Energy, LLC, was formed to enable the cooperative to provide an even higher level of service to members. Through the subsidiary, our team can leverage sustainable resources that aid in the economic viability of member-owned – and Seven States enabled – distributed generation and renewable technology projects across the Tennessee Valley.

Additional information about Seven States Power can be found on its website at www.7SPC.com.

# ABOUT LPCs

Currently, the 153 LPCs purchase all of their power requirements from TVA and distribute it to more than 10 million consumers in seven Southeastern states. These 104 municipal utilities and 49 electric cooperatives serve approximately 80,000 square miles in Tennessee, Alabama, Mississippi, Kentucky, Georgia, Virginia and North Carolina.

# INTRODUCTION

This Request for Proposals (RFP) is being issued to elicit proposals for the engineering design of a middle mile fiber optic backhaul network.  The successful bidder will submit a proposal for a complete design package interconnecting all 153 LPCs across the seven-states region.

## Project Scope

The middle mile fiber route will connect the 153 LPCs whose addresses are in Appendix A.  Please note these addresses represent corporate addresses for purposes of responding to this request and not necessarily interconnection points.

The successful bidder will be expected to meet with each LPC to finalize the appropriate location to establish an interconnection point as well as confirm other details and determine system compatibilities required for a final design. Design should include specifications for individual LPCs to ensure optimum overall network performance.

The high-level connectivity map (Appendix B) reflects the 153 LPCs within the scope of the project.

### Fiber Installation Design

The successful bidder is expected to design the network by utilizing LPC existing fiber to the greatest extent possible to reduce project costs and improve the timeline to network deployment.

Overhead Fiber Construction Requirements

- All new overhead fiber construction is preferred to be 288-count single mode fiber optical cable in accordance with industry standards

  - Preference should be given to adopt to cable standards in each LPCs service territory

  - See Appendix C for cable specifications

- All new overhead fiber construction should be placed in the communication space or the power space based on specific LPC preference

  - Placement shall be on all structure types including wood, steel, fiberglass and concrete poles

  - Cable placement to be in strict accordance with manufacture’s installation guidelines (i.e., Corning Cable Systems SRP-005-010)

- All make ready work should be based on NESC requirements and documented as part of the design.

Underground Fiber Construction Requirements

- All new underground fiber construction is preferred to be 288-count single mode fiber optical cable in accordance with industry standards

- Tracer wire shall be placed in accordance with LPC standards and requirements

- All new underground fiber construction shall use existing conduit paths or duct bank when available

  - Otherwise, new conduit duct path to be installed (no direct bury cable)

General Construction Requirements

- Ties to existing fiber construction to be completed using closures specified by LPC

- New fiber construction to be installed within an LPC’s existing rights-of-way on existing utility poles or within existing or new conduit

- Create redundant fiber rings to provide high network availability over the network

- Assume existing fiber has adequate capacity

  - Twelve fibers preferred but not required to participate in the project

- Identify and include in the design any regeneration or amplification sites necessary throughout the project

  - Specifically, applicable to fiber routes with distances greater than 70km.

Electronics Design

- The design shall include the necessary equipment required for each

  - LPC interconnection point

  - Aggregation site

  - Core site

- Equipment to include racking, electronics and interconnection materials

- The design shall include location and square footage needed for

  - Core Sites

    - DWDM transport system

    - Carrier grade routing system

  - Aggregation Sites

    - DWDM transport system

    - Carrier grade routing system

  - LPC Interconnection Points

    - Carrier grade edge system

- The design shall include building layout and size

  - Preference should be given to installing equipment at existing LPC facilities when possible

  - Climate controlled prefabricated buildings are acceptable when existing facilities are not available

  - Core sites and aggregation sites to have two entrances in order to establish security through separation of access points

- Architecture should be cost effective, flexible and optimized for long term use

- All racks should be able to accommodate standard 23-inch equipment for flexibility

- Racks at core and aggregation sites should be at least 33 inches or greater depth (internal depth) to accommodate the latest generation of routers from equipment vendors

- Electronics must be flexible and able to handle a wide range of communication-based applications

  - For packet switched services, suggested equipment shall be Carrier Ethernet 3.0 certified

- Must prioritize reliability, redundancy and resiliency

- Create aggregation points at hub sites located at designated LPCs to house core network electronics and aggregate fiber connections from the surrounding LPCs

- Create 3-4 core sites (explain if different quantity is needed) at LPCs based on geographical diversity

- Establish regional hub sites to aggregate network traffic from the expanded network

  - These hub sites will house network aggregation switches that will connect to each LPC at 10 Gbps with the ability to upgrade multiple 100 Gbps.

  - The hub sites will also house the core DWDM electronics to connect the hub sites to the core locations across the seven-states region

- The network will provide Layer 2 Ethernet services to sites using MPLS to segment traffic (the following figure is a logical network diagram)

- Access electronics will be installed at each LPC that is not an aggregation site or core site

  - The electronics will be initially configured to provide a 10 Gbps connection back to the closest aggregation site

  - The electronics can be upgraded in the future to multiple 100 Gbps circuits as demand increases on the network.

- The aggregation switches at the hubs will aggregate the network traffic from the expanded sites and connect back to the platform over 100 Gigabit Ethernet

- Core and hub locations to be connected via dense wave division multiplexing (DWDM)

- Equipment to be mounted securely in racks and cabinets in compliance with federal, State and local codes.

- Prefab shelters and/or data rooms to be provided by the LPC

## Availability of Information

The selected engineering firm will have the following information available through Seven States Power and its members.

- Access to Seven States Power staff as needed to assist with coordination and research requests.

- Access to LPC resources to identify optimal routes, interconnection points and existing fiber infrastructure.

## Project Timing

Seven States Power intends to award the contract in June 2021.  The 153 LPCs are grouped in seven regional districts.  The project will be designed and constructed in 3 phases.  Phase 1 includes the Appalachian district (23 LPCs) and the Southeastern district (16 LPCs).  Phase 2 is comprised of the Kentucky district (13 LPCs), the Central district (26 LPCs) and the North Alabama district (25 LPCs).  Phase 3 includes the Western district (22 LPCs) and the Northeast Mississippi district (28 LPCs).

Each bidder shall provide an estimated timeline for designing each phase of the project.

Timeline milestones will be negotiated at a later date for each phase of the project.

The successful bidder must assign a project manager that provides schedules and monthly status reports to Seven States Power in addition to coordinating major milestones.

## Contract Requirements

If, upon evaluation of the responses, Seven States Power decides to award a contract, Seven States Power intends to implement a contractual agreement with the successful bidder.  Seven States Power invites each bidder to submit a sample contract with its proposal.  Seven States Power reserves the right, in its sole discretion, to use bidder’s sample contract, to modify bidder’s sample contract, or to use a contract provided by legal counsel for Seven States Power.

# RFP INSTRUCTIONS

This section provides instructions for bidders preparing responses to the proposal.   Bidders are encouraged to follow these instructions as closely as possible to ensure that the Seven States Power evaluation team correctly interprets their responses.

## Contact information

Seven States Power has composed a selection team to spearhead the dialog with bidders.  Bidders will have multiple opportunities to ask questions throughout the response period.  All questions must be submitted in writing to Seven States Power’s designated contact.  The designated contact for this proposal is:

Clint Wilson

SVP, Engineering & Market Innovation

Phone 423-718-1889

E-mail: cwilson@7SPC.com

Seven States Power will respond to Bidders’ questions in writing and will distribute all questions and answers to all bidders in order to maintain a level playing field.

## Preparing Response

Compliant responses to this RFP must complete the following items:

- Non-disclosure Agreement

- Bidder Conference Call

- Formal Written RFP Response

- Customer References Review

### Non-disclosure Agreement (NDA)

Bidders interested in submitting a proposal must complete the non-disclosure agreement form in Appendix D.  Once Seven States receives the executed NDA, information will be sent to the bidder to participate in the Bidders Conference Call.

### Bidders Conference Call

Bidders that complete an NDA will have the opportunity to participate in a Bidders Conference Call to ask questions to the Middle Mile Evaluation Team.  The call will not be recorded nor will transcripts be available post call so bidders are encouraged to participate during the designated time.

### Formal Written RFP Response

This section will contain the bidder’s responses to the direct requirements listed in this RFP.  The response will also contain project approach, written cost and contract information for any related services associated with proposed work.  Prior to awarding the contract, Seven States Power reserves the right to hold responses for a period of time not to exceed ninety (90) days from the proposal due date in order to evaluate the proposals or to investigate the Bidders’ qualifications.  Each Bidder shall state in its response that its proposal, including all price terms, shall remain effective for at least ninety (90) days after the proposal due date.

### Customer References Review

Seven States Power will request three (3) references from customers who have utilized bidder’s services in similar project engagements, both in terms of project size and complexity.  Seven States Power will review references from the perspective of satisfying customer’s project objectives in terms of time, cost and quality for during and post-implementation phases.  In addition to the 3 references, Seven States Power will request a complete customer list in tabular format with minimal information.

## RFP Evaluation Timeline

Seven States Power has established the following timeline to manage the RFP process with the intent of selecting a project bidder by June 1, 2021.

| RFP Distribution | 3/1/2021 |
| --- | --- |
| Non-disclosure Agreements Returned | 3/4/2021 |
| Vendor Conference Call | 3/8/2021 |
| Vendor Question Submission Deadline | 3/15/2021 |
| RFP Response Due | 4/12/2021 |
| Customer References Review if required | 4/12/2021 – 4/30/2021 |
| Targeted Award Date | 6/1/2021 |

## Critical Evaluation Factors

Bidders should note that Seven States Power is looking for proposals for a complete engineering design for its fiber optic backhaul network.

In addition to the general feasibility of the Bidder's design in relation to the functional requirements of this specification, Seven States Power shall use the following evaluation criteria (not in any specific order):

- Bidder must have at least 5 years of experience in designing interconnected fiber optic backhaul networks.

- Bidder's performance on recent projects similar to the Seven States Power project. Bidders must provide at least three verifiable references of on-time delivery of projects of similar scope to Seven States Power within the last five years.

- Delivery schedule (must meet or exceed Seven States Power’s requirements).

- Bidder's understanding of and responsiveness to Seven States Power requirements as evidenced by the proposal and other presentations.

- Capability and availability of Bidder's staff for this project.

- Proven experience in meeting the needs of local power companies.

- Other factors at Seven States Power’s discretion.

Seven States Power encourages bidders to highlight in their responses how their proposed solutions meet the above evaluation factors.

After receipt of the proposals and completion of the review and evaluation process, Seven States Power will recommend the bidder who, in the judgment of Seven States Power, will provide the highest quality services consistent with the needs of Seven States Power.  Notwithstanding any other provisions of this RFP, Seven States Power reserves the right to award this contract to the Bidder that best meets the requirements of the RFP and not necessarily to the lowest cost bidder.  Seven States Power further reserves the right to accept or reject any or all proposals, to waive any irregularities, and to make all awards in its best interest.

## Requesting Support

Seven States Power recognizes the complexity and effort necessary to bid on this project.  Seven States Power is also committed to help bidders portray their systems and solution concepts in the best possible light.  As such, bidders are encouraged to contact Seven States Power’s designated representative with any questions relating to this RFP and the preparation of their responses.

### Ground rules

Bidders must recognize the following ground rules:

- Seven States Power will provide support on a best effort basis.

- All communications should be directed to Seven States Power’s designated contact as listed.

- Bidders must submit all questions in writing (electronic), and Seven States Power will respond to submitted questions in writing (electronic) to all bidders that execute an NDA.

# BIDDER INFORMATION

## Bidder Background

Please provide the following background information with your proposal:

- A description of your company’s size and organizational structure.

- Identification of the individual(s) to conduct negotiations on behalf of the engineering firm including individuals name, address, phone number, email, etc.

- A description of the engineering firm, its history and the services offered.

- A description of your company’s in-house support structure for the design services being proposed.

- Resumes of potential project team members

## Focus and Mission

- What is your organization’s mission?

- What is the core focus of your company?

- What is the primary differentiator you offer?

## Client Base and References

Provide customer references of three (3) existing customers with characteristics similar to Seven States Power’s project requirements.   Include a brief description of the companies and the solutions you provided.

In addition to the 3 references described above, please provide a complete list of customers using your design services.  The list should include the name of the utility, implementation date, and contact information.

*Please note that Seven States Power may contact all customer references or possibly schedule a visit if appropriate.

# RESPONSIBILITIES OF INVOLVED PARTIES

This section outlines the expectations incumbent upon both the bidders and customers during the design and implementation phases of the contract.  The intent is to provide a high-level picture of the general expectations of both parties.  The technical requirements appear in later sections of this specification.

## The Bidder’s Responsibilities

The Bidder shall be responsible for gathering information from the LPCs to complete an engineering design of an interconnect middle mile fiber backhaul network across the seven-states region.  The bidder will deliver the complete design with all documentation, drawings, and GIS records to Seven States Power as a complete engineering design package.

## Customer’s Responsibilities

During the implementation of middle mile network engineering design, Seven States Power will endeavor to provide the Bidder with all required information on a timely basis.  Seven States Power will facilitate discussions between the successful bidder and the LPCs to gather necessary information to complete the design process.

## PROJECT COORDINATION

This section describes requirements for project coordination, project meetings, project progress reporting, approvals, as well as project documentation.

## Project Team

As part of the proposal, the Bidder shall describe the general project organization related to project support and control, and the project organization intended to support the Seven States Power project.

The Seven States Power project manager shall coordinate all Seven States Power activities on the project. All communications between Seven States Power and the Bidder shall be made through the Bidder's and the Seven States Power project manager.

## Schedule

The Bidder shall submit in the proposal an overall schedule for major project activities from the time of contract award to the delivery of final products.

Within two (2) weeks after contract award and thereafter as part of each monthly progress report the Bidder shall submit to Seven States Power a detailed project schedule for Phase 1 and an estimated schedule for phases 2 and 3.

The Bidder shall explain any deviations from the planned delivery schedule and a proposed plan for recovery of the planned schedule.

## Progress Reviews

### Project Meetings

The Bidder shall schedule and participate in progress review meetings tentatively every week. Progress meetings shall be used to review the progress report, action items, and technical issue review.

### Monthly Progress Reports

A progress report shall be prepared by the Bidder prior to first scheduled project meeting of the month and be available to Seven States Power prior to each progress review meeting (once every month). The progress report shall contain the following:

- An updated detailed project schedule, with explanations for any changes.

- A detailed list of all activities completed since the previous report.

- A detailed list of activities not accomplished for the months and the reasons and contingency plans to recover.

- The status of unresolved contract changes and issues.

- A list of current or anticipated problem areas and proposed solutions, if any.

- Updated lists of Bidder and Seven States Power action items along with required resolution dates.

# PRICING REQUIREMENTS

This section describes the pricing requirements for the RFP.  The bidder must complete pricing tables in this section.

The detailed design shall provide a detailed cost estimate for the buildout of the middle mile fiber optic backhaul network.  Fiber construction scenarios are required with a cost breakout per mile for each scenario.  Provide a schedule for the different scenarios with mileage required, cost per unit and total cost.

Costs should include totals for

- Connecting all LPCs

- Costs totaled by state

- Costs totaled by districts (see Appendix A)

Fiber Construction Scenarios

- The table below is a suggested guideline but other scenarios may be needed

- New fiber construction to use existing pole lines

- Assume LPCs complete their own make ready work

| Fiber Construction Scenarios | Miles | % Fiber Construction Needed | Cost per Mile | Subtotal |
| --- | --- | --- | --- | --- |
| Middle-mile routes overlapping existing routes from LPCs |  |  |  |  |
| New middle-mile routes |  |  |  |  |
| Middle-mile redundancy routes |  |  |  |  |
| Middle-mile redundancy routes overlapping existing routes |  |  |  |  |
| Total middle-mile routes (including redundancy routes) |  |  |  |  |

Network Electronics Estimate

| Network Electronics | Quantity | Cost | Subtotal | Annual Maintenance |
| --- | --- | --- | --- | --- |
| Core Sites |  |  |  |  |
| - DWDM Transport Equipment |  |  |  |  |
| - Core Routing Equipment |  |  |  |  |
| DWDM / Aggregation Sites |  |  |  |  |
| - DWDM Transport Equipment |  |  |  |  |
| - Aggregation Routing Equipment |  |  |  |  |
| LPCs |  |  |  |  |
| - Edge Routing/Switching Equipment |  |  |  |  |

Maintenance and Operating Cost Estimates

| Maintenance and Operating Costs | Unit | Unit Cost | Subtotal |
| --- | --- | --- | --- |
| Fiber Maintenance |  |  |  |
| Equipment Maintenance Contracts |  |  |  |
| NOC Fee |  |  |  |
| Network Engineer |  |  |  |
| Network Manager |  |  |  |
| Network Technician |  |  |  |
| Equipment Replacement Fund (7-year lifecycle) |  |  |  |
| Incidentals |  |  |  |
| Total Maintenance and Operating Costs |  |  |  |

Alternate Pricing

- NOC services (yearly cost)

  - Create and provide specifications to allow 7SPC to RFP NOC services

    - Trouble ticket management

    - Monitor health of network

    - Escalate issues to the proper operations groups

    - Network engineering

    - Manage network electronics

    - Etc.

# CONFIDENTIALITY

All proposals submitted will be deemed confidential and will not be available for review by anyone other than Seven States Power staff and membership or authorized agents or representatives of Seven States Power.

# Deliverable Requirements

This section describes the deliverable requirements of the project.  The proposal shall be judged by its conformance to the functional requirements of this section.

- Deliverable to be a fiber network construction package (specifications and construction prints) that can be used to bid fiber construction and network equipment purchases and build out

- Construction Bids shall require:

  - Engineering: Includes system-level architecture planning, preliminary designs, and engineering field walk-outs to determine candidate fiber routing; development of detailed engineering prints and preparation of permit applications; and post-construction “as-built” revisions to engineering design materials.

  - Project Management / Quality Assurance: Includes expert quality assurance field review of final construction for acceptance, review of invoices, tracking progress, and coordination of field changes.

  - General Outside Plant Construction Labor: Consists of all labor related to “typical” outside plant construction, including conduit placement, fiber installation, and surface restoration; includes all work area protection and traffic control measures inherent to all roadway construction activities.

  - General Outside Plant Construction Materials: Consists of all materials related to “typical” outside plant construction, including new aerial messenger strand, attachment hardware (if appliable), conduit, handholes, fiber optics, and splicing materials.

  - Railroad, Bridge, and Interstate Crossings: Consists of specialized engineering, permitting, and incremental construction.

  - Outside Plant Fiber Splicing: Includes all labor related to fiber splicing of outdoor fiber optic cables.

  - Fiber Termination/Building Entrance: Consists of all costs related to fiber lateral installation into network sites, including outside plant construction on private property, building penetration, inside plant construction to a typical backbone network service “demarcation” point, fiber termination, and fiber testing.

  - The successful bidder will be required to include consistent GIS data recording and mapping included with construction drawings.

  - The successful bidder will be expected to perform a physical walkout recording GIS plot points to identify conduit routes for design (for underground), routes for design, route planning and protection.

Appendix A

LPC Corporate Addresses by District

| LPC Corporate Addresses Listed by District |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Phase | Member District | LPC Name | Meters | Street Address | City | State | Zip |
| 1 | 1 | Appalachian District | Knoxville Utilities Board | 206,019 | 445 S. Gay Street | Knoxville | TN | 37902-1109 |
| 2 | 1 | Appalachian District | BrightRidge | 79,146 | 2600 Boones Creek Road | Johnson City | TN | 37615 |
| 3 | 1 | Appalachian District | Lenoir City Utilities Board | 67,688 | 7698 Creekwood Park Blvd | Lenoir City | TN | 37772 |
| 4 | 1 | Appalachian District | Sevier County Electric System | 55,702 | 315 East Main Street | Sevierville | TN | 37862 |
| 5 | 1 | Appalachian District | Appalachian Electric Cooperative | 47,135 | 1109 Hill Drive | New Market | TN | 37820 |
| 6 | 1 | Appalachian District | Greeneville Light & Power System | 38,498 | 110 North College Street | Greeneville | TN | 37743 |
| 7 | 1 | Appalachian District | Mountain Electric Cooperative, Inc. | 34,348 | 604 South Church Street | Mountain City | TN | 37683 |
| 8 | 1 | Appalachian District | Bristol Tennessee Essential Services | 33,684 | 2470 Volunteer Parkway | Bristol | TN | 37620 |
| 9 | 1 | Appalachian District | Powell Valley Electric Cooperative | 32,390 | 420 Straight Creek Road | New Tazewell | TN | 37825 |
| 10 | 1 | Appalachian District | Holston Electric Cooperative | 30,542 | 1200 West Main Street | Rogersville | TN | 37857 |
| 11 | 1 | Appalachian District | Clinton Utilities Board | 29,970 | 1001 Charles G. Seivers Blvd. | Clinton | TN | 37716 |
| 12 | 1 | Appalachian District | City of Alcoa Electric Department | 29,779 | 725 Universal Street | Alcoa | TN | 37701 |
| 13 | 1 | Appalachian District | City of Elizabethton Electric Department | 26,449 | 400 Hatcher Lane | Elizabethton | TN | 37643 |
| 14 | 1 | Appalachian District | LaFollette Utilities Board | 22,220 | 302 North Tennessee Avenue | LaFollette | TN | 37766 |
| 15 | 1 | Appalachian District | Newport Utilities Board | 21,887 | 170 Cope Boulevard | Newport | TN | 37821 |
| 16 | 1 | Appalachian District | Maryville Electric Department | 21,820 | 332 Home Avenue | Maryville | TN | 37801-3971 |
| 17 | 1 | Appalachian District | Plateau Electric Cooperative | 17,047 | 16200 Scott Hwy. | Oneida | TN | 37841 |
| 18 | 1 | Appalachian District | BVU Authority | 16,326 | 15022 Lee Highway | Bristol | VA | 24202 |
| 19 | 1 | Appalachian District | Oak Ridge Electric Department | 16,321 | 100 Woodbury Lane | Oak Ridge | TN | 37830 |
| 20 | 1 | Appalachian District | Morristown Utilities Commission | 15,027 | 441 West Main Street | Morristown | TN | 37814 |
| 21 | 1 | Appalachian District | Harriman Utility Board | 10,943 | 300 N Roane Street | Harriman | TN | 37748 |
| 22 | 1 | Appalachian District | Erwin Utilities | 8,916 | 244 Love Street | Erwin | TN | 37650 |
| 23 | 1 | Appalachian District | Jellico Utilities Authority | 4,459 | 410 South Main Street | Jellico | TN | 37762 |
| 24 | 1 | Southeastern District | Electric Power Board of Chattanooga | 187,010 | 10 W. ML King Blvd. | Chattanooga | TN | 37402 |
| 25 | 1 | Southeastern District | Volunteer Energy Cooperative | 117,886 | 18359 Hwy. 58 North | Decatur | TN | 37322 |
| 26 | 1 | Southeastern District | North Georgia Electric Membership Corporation | 100,809 | 1850 Cleveland Highway | Dalton | GA | 30721 |
| 27 | 1 | Southeastern District | Blue Ridge Mountain Electric Membership Corporation | 53,578 | 875 Main Street East | Young Harris | GA | 30582 |
| 28 | 1 | Southeastern District | Sequachee Valley Electric Cooperative | 35,669 | 512 South Cedar Avenue | South Pittsburg | TN | 37380 |
| 29 | 1 | Southeastern District | Fort Loudoun Electric Cooperative | 33,397 | 116 Tellico Port Road | Vonore | TN | 37885 |
| 30 | 1 | Southeastern District | Cleveland Utilities | 33,345 | 2450 Guthrie Avenue, NW | Cleveland | TN | 37311 |
| 31 | 1 | Southeastern District | Tri-State Electric Membership Corporation | 19,440 | 2310 Blue Ridge Drive | McCaysville | GA | 30555 |
| 32 | 1 | Southeastern District | Rockwood Electric Utility | 14,689 | 341 West Rockwood Street | Rockwood | TN | 37854 |
| 33 | 1 | Southeastern District | Athens Utilities Board | 13,385 | 100 Englewood Road | Athens | TN | 37303 |
| 34 | 1 | Southeastern District | Loudon Utilities | 12,390 | 201 Alma Place | Loudon | TN | 37774 |
| 35 | 1 | Southeastern District | City of Dayton Electric Department | 10,746 | 421 First Avenue | Dayton | TN | 37321 |
| 36 | 1 | Southeastern District | Sweetwater Utilities Board | 9,100 | 101 Oak Street | Sweetwater | TN | 37874 |
| 37 | 1 | Southeastern District | Etowah Utilities Department | 5,105 | 1313 South Tennessee Avenue | Etowah | TN | 37331 |
| 38 | 1 | Southeastern District | Murphy Power Board | 5,090 | 107 Peachtree Street | Murphy | NC | 28906 |
| 39 | 1 | Southeastern District | Chickamauga Electric System | 982 | 103 Crittenden Avenue | Chickamauga | GA | 30707 |
| 40 | 2 | Central District | Nashville Electric Service | 408,812 | 1214 Church Street | Nashville | TN | 37246 |
| 41 | 2 | Central District | Middle Tennessee Electric Membership Corporation | 299,428 | 555 New Salem Hwy | Murfreesboro | TN | 37129 |
| 42 | 2 | Central District | Cumberland Electric Membership Corporation | 98,404 | 1940 Madison Street | Clarksville | TN | 37043 |
| 43 | 2 | Central District | Duck River Electric Membership Corporation | 76,560 | 1411 Madison Street | Shelbyville | TN | 37160 |
| 44 | 2 | Central District | CDE Lightband (Clarksville) | 70,960 | 2021 Wilma Rudolph Boulevard | Clarksville | TN | 37040 |
| 45 | 2 | Central District | Tri-County Electric Membership Corporation | 52,873 | 405 College Street | Lafayette | TN | 37083 |
| 46 | 2 | Central District | Upper Cumberland Electric Membership Corporation | 50,478 | 907 Main Street N | Carthage | TN | 37030-1855 |
| 47 | 2 | Central District | Dickson Electric System | 35,555 | 236 Cowan Road | Dickson | TN | 37055 |
| 48 | 2 | Central District | Meriwether Lewis Electric Cooperative | 34,411 | 1625 Highway 100 | Centerville | TN | 37033 |
| 49 | 2 | Central District | Caney Fork Electric Cooperative, Inc. | 32,699 | 920 Smithville Highway | McMinnville | TN | 37110 |
| 50 | 2 | Central District | Columbia Power & Water Systems | 29,876 | 201 Pickens Lane | Columbia | TN | 38401 |
| 51 | 2 | Central District | Gallatin Department of Electricity | 21,148 | 135 Jones Street | Gallatin | TN | 37066 |
| 52 | 2 | Central District | Lawrenceburg Utility Systems | 20,540 | 1607 North Locust Avenue | Lawrenceburg | TN | 38464 |
| 53 | 2 | Central District | Tennessee Valley Electric Cooperative | 19,555 | 590 Florence Road | Savannah | TN | 38372 |
| 54 | 2 | Central District | Fayetteville Public Utilities | 18,616 | 408 College Street West | Fayetteville | TN | 37334 |
| 55 | 2 | Central District | Cookeville Electric Department | 18,452 | 55 West Davis Road | Cookeville | TN | 38506 |
| 56 | 2 | Central District | Pulaski Electric System | 14,571 | 128 South First Street | Pulaski | TN | 38478 |
| 57 | 2 | Central District | Tullahoma Utilities Authority | 10,926 | 901 South Jackson | Tullahoma | TN | 37388 |
| 58 | 2 | Central District | Shelbyville Power System | 10,411 | 308 South Main Street | Shelbyville | TN | 37160 |
| 59 | 2 | Central District | Springfield Department of Electricity | 8,369 | 1000 Central Ave West | Springfield | TN | 37172 |
| 60 | 2 | Central District | McMinnville Electric System | 8,062 | 200 Morford Street | McMinnville | TN | 37110 |
| 61 | 2 | Central District | Lewisburg Electric System | 6,264 | 599 West Ellington Parkway | Lewisburg | TN | 37091 |
| 62 | 2 | Central District | Winchester Utility System | 6,144 | 219 Second Avenue NW | Winchester | TN | 37398 |
| 63 | 2 | Central District | Mount Pleasant Power System | 4,057 | 123 North Main Street | Mt. Pleasant | TN | 38474 |
| 64 | 2 | Central District | Sparta Electric & Public Works | 2,983 | 545 East Brockman Way | Sparta | TN | 38583 |
| 65 | 2 | Central District | Smithville Electric System | 2,788 | 611 East Broad Street | Smithville | TN | 37166 |
| 66 | 2 | Kentucky District | Warren Rural Electric Cooperative Corporation | 67,376 | 951 Fairview Avenue | Bowling Green | KY | 42101 |
| 67 | 2 | Kentucky District | Pennyrile Rural Electric Cooperative Corporation | 44,112 | 2000 Harrison Street | Hopkinsville | KY | 42240 |
| 68 | 2 | Kentucky District | West Kentucky Rural Electric Cooperative Corporation | 38,884 | 1218 West Broadway | Mayfield | KY | 42066 |
| 69 | 2 | Kentucky District | Bowling Green Municipal Utilities | 30,952 | 801 Center Street | Bowling Green | KY | 42101 |
| 70 | 2 | Kentucky District | Hopkinsville Electric System | 12,856 | 1820 East Ninth Street | Hopkinsville | KY | 42240-4434 |
| 71 | 2 | Kentucky District | Murray Electric System | 7,998 | 401 Olive Street | Murray | KY | 42071 |
| 72 | 2 | Kentucky District | Glasgow Electric Plant Board | 7,453 | 100 Mallory Drive | Glasgow | KY | 42141 |
| 73 | 2 | Kentucky District | Mayfield Electric & Water System | 5,369 | 301 East Broadway | Mayfield | KY | 42066 |
| 74 | 2 | Kentucky District | Franklin Electric Plant Board | 5,152 | 309 North High Street | Franklin | KY | 42134 |
| 75 | 2 | Kentucky District | Russellville Electric Plant Board | 4,083 | 165 East Fourth Street | Russellville | KY | 42276 |
| 76 | 2 | Kentucky District | Benton Electric System | 2,522 | 436 Mayfield Highway | Benton | KY | 42025 |
| 77 | 2 | Kentucky District | Fulton Electric System | 1,607 | 501 Walnut Street | Fulton | KY | 42041 |
| 78 | 2 | Kentucky District | Hickman Electric Plant Board | 1,061 | 1104 Moscow Avenue | Hickman | KY | 42050 |
| 79 | 2 | North Alabama District | Huntsville Utilities | 188,414 | 112 Spragins St. NW | Huntsville | AL | 35801 |
| 80 | 2 | North Alabama District | City of Florence Utilities | 49,922 | 502 East Dr. Hicks Blvd. | Florence | AL | 35630 |
| 81 | 2 | North Alabama District | City of Athens Electric Department | 48,085 | 1806 Wilkinson Street | Athens | AL | 35611 |
| 82 | 2 | North Alabama District | Cullman Electric Cooperative | 44,047 | 1749 Eva Road, NE | Cullman | AL | 35055 |
| 83 | 2 | North Alabama District | Joe Wheeler Electric Membership Corporation | 43,657 | 25700 Alabama Highway 24 | Trinity | AL | 35673 |
| 84 | 2 | North Alabama District | Sand Mountain Electric Cooperative | 31,450 | 402 Main Street West | Rainsville | AL | 35986 |
| 85 | 2 | North Alabama District | Decatur Utilities | 26,590 | 1002 Central Parkway, SW | Decatur | AL | 35601 |
| 86 | 2 | North Alabama District | Cherokee Electric Cooperative | 24,159 | 1550 Clarence Chestnut ByPass | Centre | AL | 35960 |
| 87 | 2 | North Alabama District | Marshall-DeKalb Electric Cooperative | 19,979 | 10025 Highway 168 | Boaz | AL | 35957 |
| 88 | 2 | North Alabama District | Sheffield Utilities | 18,939 | 300 North Nashville Avenue | Sheffield | AL | 35660 |
| 89 | 2 | North Alabama District | North Alabama Electric Cooperative | 18,350 | 41103 US Hwy. 72 | Stevenson | AL | 35772 |
| 90 | 2 | North Alabama District | Arab Electric Cooperative, Inc. | 15,574 | 331 S. Brindlee Mtn. Pkwy. | Arab | AL | 35016 |
| 91 | 2 | North Alabama District | Bessemer Electric Service | 11,141 | 1600 1st Avenue North | Bessemer | AL | 35021 |
| 92 | 2 | North Alabama District | Albertville Municipal Utilities Board | 10,242 | 210 West Main Street | Albertville | AL | 35950 |
| 93 | 2 | North Alabama District | Cullman Power Board | 8,768 | 106 2nd Avenue NE | Cullman | AL | 35055 |
| 94 | 2 | North Alabama District | Fort Payne Improvement Authority | 8,551 | 406 3rd Street SE | Fort Payne | AL | 35967 |
| 95 | 2 | North Alabama District | Scottsboro Electric Power Board | 8,232 | 404 East Willow Street | Scottsboro | AL | 35768 |
| 96 | 2 | North Alabama District | Muscle Shoals Electric Board | 8,227 | 1015 Avalon Avenue | Muscle Shoals | AL | 35661 |
| 97 | 2 | North Alabama District | Franklin Electric Cooperative | 7,727 | 255 West Franklin Street | Russellville | AL | 35653 |
| 98 | 2 | North Alabama District | Guntersville Electric Board | 6,515 | 701 Blount Avenue | Guntersville | AL | 35976 |
| 99 | 2 | North Alabama District | Hartselle Utilities | 5,550 | 1010 Sparkman Street NW | Hartselle | AL | 35640 |
| 100 | 2 | North Alabama District | Russellville Electric Board | 5,103 | 721 Jackson Avenue South | Russellville | AL | 35653 |
| 101 | 2 | North Alabama District | Tuscumbia Electricity Department | 4,881 | 202 East 6th Street | Tuscumbia | AL | 35674 |
| 102 | 2 | North Alabama District | Tarrant Electric Department | 2,752 | 1604 Pinson Valley Parkway | Tarrant | AL | 35217 |
| 103 | 2 | North Alabama District | Courtland Electric Department | 781 | 361 College Street | Courtland | AL | 35618 |
| 104 | 3 | Northeast Mississippi District | 4-County Electric Power Association | 48,897 | 5265 South Frontage Road | Columbus | MS | 39701 |
| 105 | 3 | Northeast Mississippi District | Tombigbee Electric Power Association | 43,929 | 1346 Auburn Road | Tupelo | MS | 38804 |
| 106 | 3 | Northeast Mississippi District | Central Electric Power Association | 36,677 | 107 East Main Street | Carthage | MS | 39051 |
| 107 | 3 | Northeast Mississippi District | Northcentral Mississippi Electric Power Association | 32,158 | 4600 Northcentral Way | Olive Branch | MS | 38654 |
| 108 | 3 | Northeast Mississippi District | Tallahatchie Valley Electric Power Association | 27,546 | 250 Power Drive | Batesville | MS | 38606 |
| 109 | 3 | Northeast Mississippi District | North East Mississippi Electric Power Association | 26,968 | 10 PR 2050 | Oxford | MS | 38655 |
| 110 | 3 | Northeast Mississippi District | Pontotoc Electric Power Association | 19,457 | 12 South Main Street Materials - 930 Hwy 41 South | Pontotoc | MS | 38863 |
| 111 | 3 | Northeast Mississippi District | Alcorn County Electric Power Association | 18,829 | 1909 S. Tate Street | Corinth | MS | 38834 |
| 112 | 3 | Northeast Mississippi District | Natchez Trace Electric Power Association | 15,828 | 555 East Madison Street | Houston | MS | 38851 |
| 113 | 3 | Northeast Mississippi District | City of Tupelo Light & Water | 15,184 | 320 North Front Street | Tupelo | MS | 38804 |
| 114 | 3 | Northeast Mississippi District | Tippah Electric Power Association | 13,825 | 109 East Cooper Street | Ripley | MS | 38663 |
| 115 | 3 | Northeast Mississippi District | Prentiss County Electric Power Association | 13,818 | 302 West Church Street | Booneville | MS | 38829 |
| 116 | 3 | Northeast Mississippi District | Starkville Utilities | 13,578 | 200 N Lafayette | Starkville | MS | 39759 |
| 117 | 3 | Northeast Mississippi District | Tishomingo County Electric Power Association | 13,515 | 205 Constitution Drive | Iuka | MS | 38852 |
| 118 | 3 | Northeast Mississippi District | East Mississippi Electric Power Association | 12,828 | 2128 Highway 39 North | Meridian | MS | 39302 |
| 119 | 3 | Northeast Mississippi District | Columbus Light & Water | 12,767 | 420 4th Avenue S. | Columbus | MS | 39701 |
| 120 | 3 | Northeast Mississippi District | Holly Springs Utilities Department | 11,630 | 1050 East Highway 4 | Holly Springs | MS | 38635 |
| 121 | 3 | Northeast Mississippi District | Monroe County Electric Power Association | 11,255 | 601 North Main | Amory | MS | 38821 |
| 122 | 3 | Northeast Mississippi District | New Albany Light, Gas & Water | 10,881 | 100 Cleveland | New Albany | MS | 38652 |
| 123 | 3 | Northeast Mississippi District | Oxford Utilities | 9,661 | 300 McElroy Drive | Oxford | MS | 38655 |
| 124 | 3 | Northeast Mississippi District | City of Okolona Electric Department | 5,300 | 209 West Main Street | Okolona | MS | 38860 |
| 125 | 3 | Northeast Mississippi District | Amory Water & Electric | 4,013 | 129 Main Street N | Amory | MS | 38821 |
| 126 | 3 | Northeast Mississippi District | West Point Electric System | 3,924 | 644 East Broad Street | West Point | MS | 39773 |
| 127 | 3 | Northeast Mississippi District | Philadelphia Utilities | 3,830 | 435 East Myrtle Street | Philadelphia | MS | 39350 |
| 128 | 3 | Northeast Mississippi District | Louisville Utilities | 3,437 | 871 South Columbus Avenue | Louisville | MS | 39339 |
| 129 | 3 | Northeast Mississippi District | Aberdeen Electric Department | 3,228 | 612 West Commerce | Aberdeen | MS | 39730 |
| 130 | 3 | Northeast Mississippi District | City of Water Valley Electric Department | 2,044 | 101 Blackmur Drive | Water Valley | MS | 38965 |
| 131 | 3 | Northeast Mississippi District | Macon Electric Department | 1,192 | 2943 Jefferson Street | Macon | MS | 39341 |
| 132 | 3 | Western District | Memphis Light, Gas & Water Division | 414,779 | 220 South Main Street | Memphis | TN | 38103-3917 |
| 133 | 3 | Western District | Southwest Tennessee Electric Membership Corporation | 50,336 | 1009 East Main Street | Brownsville | TN | 38012 |
| 134 | 3 | Western District | Gibson Electric Membership Corporation | 38,210 | 1207 South College Street | Trenton | TN | 38382 |
| 135 | 3 | Western District | Jackson Energy Authority | 36,231 | 250 North Highland | Jackson | TN | 38301 |
| 136 | 3 | Western District | Lexington Electric System | 22,483 | 92 South Main Street | Lexington | TN | 38351 |
| 137 | 3 | Western District | Paris Board of Public Utilities | 21,395 | 117 East Washington Street | Paris | TN | 38242 |
| 138 | 3 | Western District | Pickwick Electric Cooperative | 20,614 | 672 Hwy 142 | Selmer | TN | 38375 |
| 139 | 3 | Western District | Weakley County Municipal Electric System | 20,546 | 11181 Hwy 22 | Martin | TN | 38237 |
| 140 | 3 | Western District | Chickasaw Electric Cooperative | 20,409 | 17970 Highway 64 East | Somerville | TN | 38068 |
| 141 | 3 | Western District | Carroll County Electric Department | 15,429 | 103 West Paris Street | Huntingdon | TN | 38344 |
| 142 | 3 | Western District | Dyersburg Electric System | 12,052 | 211 East Court Street | Dyersburg | TN | 38024 |
| 143 | 3 | Western District | Bolivar Energy Authority | 11,137 | 815 Tennessee Street | Bolivar | TN | 38008 |
| 144 | 3 | Western District | Benton County Electric System | 10,236 | 975 Hwy 70 East | Camden | TN | 38320 |
| 145 | 3 | Western District | Forked Deer Electric Cooperative | 9,948 | 1135 North Church Street | Halls | TN | 38040 |
| 146 | 3 | Western District | Milan Department of Public Utilities | 8,195 | 1085 South Second Street | Milan | TN | 38358 |
| 147 | 3 | Western District | Ripley Power & Light | 6,770 | 150 South Main | Ripley | TN | 38063 |
| 148 | 3 | Western District | Union City Energy Authority | 6,442 | 312 North Division Street | Union City | TN | 38261 |
| 149 | 3 | Western District | Brownsville Utility Department | 5,331 | 25 North Lafayette | Brownsville | TN | 38012 |
| 150 | 3 | Western District | Covington Electric System | 4,710 | 1469 South Main Street | Covington | TN | 38019 |
| 151 | 3 | Western District | Humboldt Utilities | 4,455 | 207 South 13th Avenue | Humboldt | TN | 38343 |
| 152 | 3 | Western District | Trenton Light & Water Department | 2,511 | 109 West Armory Street | Trenton | TN | 38382 |
| 153 | 3 | Western District | Newbern Electric Water & Gas | 1,814 | 302 East Main Street | Newbern | TN | 38059 |

Appendix B

LPC Map with Potential Fiber Routes

Appendix C

Fiber Optic Cable Specifications

Specification for Fiber Optic Cable and Micro cable

- General Information

This following specifies the requirements for fiber optic loose tube cable and micro cable.  These cable designs incorporate gel-filled loose buffer tubes with dry waterblocking yarns that are stranded via the reverse oscillating lay method around a dielectric central strength member. The core will be made with up to 24 buffer tubes. The core is surrounded by dielectric strength elements and an outer polyethylene jacket to provide abrasion resistance, overall protection, and assist with the blowing process.   For versions with Light Armor and Armor, electrolytically chrome-coated steel (ECCS) steel armor tape will surround the cable core.

Rationale:  This section of the specification highlights the general properties of the cable.

- Cable Design and Application Specifics

All loose tube cables supplied must meet the requirements of RDUP 7 CFR 1755.900, Telcordia GR-20-CORE Issue 4, ANSI/ICEA S-87-640-2005.  All micro cables must comply with the requirements for Micro-Duct cable from Telcordia GR-20-CORE, Issue 4.  The RDUP Mid-span test is N/A for micro cables.

Short-term and long-term tensile ratings

| Cable type | Fiber count | Short term tension– lbs (N) | Crush lb/in (N/cm) |
| --- | --- | --- | --- |
| Loose tube cables | All | 600 (2700) | 125 (220) |
| Micro cables | All | 300 (1334) | 125 (220) |

All testing performed must be based on EIA/TIA-455 standards. Manufacturing of this cable must be completed under strict quality control standards with ISO 9001 and TL9000 R4.0 certification. The manufacturer must be qualified to ISO 14001:2004.

Environmental temperatures ranges are specified in Table 1.

Table 1 – Recommended Environmental Temperatures

| Operating Temperature | -40 to 70°C (-40 to 158°F) |
| --- | --- |
| Installation Temperature | -30 to 70°C (-22 to 158°F) |
| Storage/Shipping Temperature | -50 to 75°C (-22 to 167°F) |

Since the fiber optic cable is a long-term investment, the Manufacturer must show production of loose tube cables for at least 20 years and provide references of deployed cables > 20 years old from the date of the RFP.

- Cable Construction

3.1	 	Central Member

The cable shall have a central member to function as an anti-buckling element.  The central member shall be constructed from a glass/epoxy composite dielectric rod. A polyethylene overcoat may be applied to the central member to provide the proper spacing between buffer tubes during stranding if required by geometric constraints.  A water blocking thread shall be placed longitudinally along the central member.

  - Buffer Tubes

The fiber buffer tubes shall be constructed of Polybutylene Teraphthalate (PBT).  Loose tube cables will have nominal buffer tube diameter of 2.5 mm.  Micro cables will have nominal buffer tube diameter of 1.7 mm. Each buffer tube shall contain up to 12 fibers.  The fibers will be loose within the buffer tubes, allowing the fibers to move freely.  Filling compound shall be utilized in the buffer tube to prevent water intrusion and migration. The filling compound shall be nontoxic and dermatologically safe to exposed skin. It shall be chemically and mechanically compatible with all cable components, non-nutritive to fungus, nonhygroscopic, and electrically nonconductive.

  - Fiber and Buffer Tube Color Code

The individual fibers and buffer tubes are color coded for ease of identification per EIA/TIA-598.  Buffer tubes 13 through 24 are marked with a black co-extruded stripe 180° apart for identification purposes.  The black tube (Tube 20) will be marked with a white co-extruded stripe.  Nominal stripe width shall be 1mm.

Table 2 – Fiber and Buffer Tube Color Code

| Fiber or Tube Number | Color | Tube Number | Color |
| --- | --- | --- | --- |
| 1 | Blue | 13 | Blue-Striped |
| 2 | Orange | 14 | Orange-Striped |
| 3 | Green | 15 | Green-Striped |
| 4 | Brown | 16 | Brown-Striped |
| 5 | Slate | 17 | Slate-Striped |
| 6 | White | 18 | White-Striped |
| 7 | Red | 19 | Red-Striped |
| 8 | Black | 20 | Black-Striped |
| 9 | Yellow | 21 | Yellow-Striped |
| 10 | Violet | 22 | Violet-Striped |
| 11 | Rose | 23 | Rose-Striped |
| 12 | Aqua | 24 | Aqua-Striped |

  - Filler Rods

In order to create a round cable, filler rods of the same diameter as the buffer tubes may be used to fill empty positions.  Filler rods should be made out of HDPE and natural in color.

  - Buffer Tube Stranding

The buffer tubes (and filler rods, if necessary) shall be stranded in a reverse oscillation lay (ROL) or “S-Z” technique around the central member for easy mid-span access.

  - Cable Core Binders

The core shall be wrapped with two counter helically applied polyester binders to bind together the cable core.  These binders may contain dry water blocking material when necessary to meet water penetration requirements.

  - Cable Core Water Blocking

Water blocking is accomplished via gel-free elements within the cable core.  Water blocking elements, binders and/or tapes depending on the cable construction, shall be applied about the cable’s core and appropriate jacket materials to provide water blocking in the interstices created between the cable materials.  Water blocking performance must comply with GR-20, section 6.3.1.1 using the test method in compliance with FOTP-82.

  - Strength Elements

Fiberglass strength elements are applied over the cable core to provide the cable with the required tensile strength.

  - Inner Jacket (for loose tube Armored only)

An inner medium density polyethylene jacket will be applied over the stranded cable core for Armored (not Light Armored) cables only. The jacket shall be continuous, free from pinholes, splits, blisters, or other imperfections and have a nominal thickness of 0.7mm.

  - Outer Jacket

Loose tube cables - A black MDPE outer polyethylene jacket is applied over the cable to provide overall mechanical protection.  The MDPE must be in compliance with ASTM D 1248: Type II, Class C, Category 4, either Grades E9 or J4. If required, the jacket shall have two co-extruded colored tracer stripes located 180 degrees apart to aid in cable identification. The jacket will be continuous, free from pinholes; splits, blisters, or other imperfections and have a nominal minimum thickness of 1.3 mm for loose tube and 0.5 mm for microcables.

Micro cables - A black HDPE outer polyethylene jacket is applied over the cable to provide overall mechanical protection. If required, the jacket shall have two co-extruded colored tracer stripes located 180 degrees apart to aid in cable identification. The jacket will be continuous, free from pinholes; splits, blisters, or other imperfections and have a nominal thickness of 0.72 mm.

  - Ripcords

    - Single Jacket

For ease of jacket removal, at least one (1) clearly identifiable polyester ripcords shall be placed underneath the outer jacket.

    - Light Armor

For ease of jacket removal, two (2) clearly identifiable aramid ripcords shall be placed underneath the armor/inner jacket layer with approximately 180 degrees of separation.

    - Armored

For ease of jacket removal, one (1) clearly identifiable polyester ripcord shall be placed underneath the inner jacket.  One aramid ripcord shall be placed under the armor/outer jacket.

  - Steel Armor (for Light Armor and Armored loose tube cables only)

    - Light Armored

A corrugated, co-polymer coated carbon steel (ECCS) tape with water blocking material shall be applied longitudinally directly over the fiberglass strength elements to provide mechanical protection against rodent damage.  The tape shall be 0.008” thick and coated on both sides for corrosion resistance, while easily removed during fiber splicing.   The armor shall have an overlapping seam.  The armor shall contain a layer of dry water blocking materials on the side of the armor that faces the cable core.

    - Armored

A corrugated, co-polymer coated carbon steel (ECCS) tape with water blocking material shall be applied longitudinally over the inner jacket to provide mechanical protection against rodent damage.  The tape shall be 0.008” thick and coated on both sides for corrosion resistance, while easily removed during fiber splicing.   The armor shall have an overlapping seam.   The armor contains a layer of dry water blocking materials on the side that faces the cable core.

  - Dimensions

Below are the specified cable configurations and dimensions.

| Fiber Count | Nominal Outer Diameter (in) | Nominal Outer Diameter (mm) | Nominal Cable Weight (lb/1000ft) | Nominal Cable Weight(kg/km) |
| --- | --- | --- | --- | --- |
| Single Jacket Loose Tube |  |  |  |  |
| 002-060 | 0.390 | 9.9 | 48 | 71 |
| 061-072 | 0.413 | 10.5 | 55 | 82 |
| 073-096 | 0.484 | 12.3 | 74 | 110 |
| 097-120 | 0.547 | 13.9 | 96 | 143 |
| 121-144 | 0.618 | 15.7 | 120 | 179 |
| 145-216 | 0.606 | 15.4 | 111 | 165 |
| 217-240 | 0.638 | 16.2 | 125 | 186 |
| 241-288 | 0.709 | 18.0 | 154 | 229 |
| 300-432 | 0.835 | 21.2 | 233 | 346 |
| Light Armor Loose Tube - Single Armor, Single Jacket: |  |  |  |  |
| 002-060 | 0.461 | 11.7 | 82 | 122 |
| 061-072 | 0.484 | 12.3 | 91 | 135 |
| 073-096 | 0.555 | 14.1 | 116 | 172 |
| 097-120 | 0.618 | 15.7 | 148 | 220 |
| 121-144 | 0.693 | 17.6 | 179 | 267 |
| 145-216 | 0.681 | 17.3 | 166 | 247 |
| 217-240 | 0.713 | 18.1 | 184 | 274 |
| 241-288 | 0.780 | 19.8 | 220 | 327 |
| 300-432 | 0.898 | 22.8 | 304 | 453 |
| Single Armor, Double Jackets |  |  |  |  |
| 002-060 | 0.520 | 13.2 | 101 | 150 |
| 061-072 | 0.547 | 13.9 | 115 | 171 |
| 073-096 | 0.614 | 15.6 | 141 | 210 |
| 097-120 | 0.681 | 17.3 | 172 | 256 |
| 121-144 | 0.752 | 19.1 | 206 | 307 |
| 145-216 | 0.740 | 18.8 | 196 | 291 |
| 217-240 | 0.772 | 19.6 | 215 | 320 |
| 241-288 | 0.843 | 21.4 | 254 | 378 |
| 300-432 | 0.957 | 24.3 | 340 | 506 |
| Double Jacket Loose Tube Dielectric - Double Jackets |  |  |  |  |
| 002-060 | 0.460 | 11.7 | 66 | 98 |
| 002-072 | 0.480 | 12.2 | 72 | 107 |
| 073-096 | 0.550 | 14.0 | 93 | 138 |
| 097-120 | 0.613 | 15.6 | 118 | 176 |
| 121-144 | 0.684 | 17.4 | 144 | 215 |
| 145-216 | 0.676 | 17.2 | 137 | 204 |
| 217-240 | 0.708 | 18.0 | 154 | 229 |
| 241-288 | 0.778 | 19.8 | 185 | 275 |
| Dielectric microcables |  |  |  |  |
| 002-072 | 0.25 | 6.4 | 24 | 35 |
| 073-096 | 0.30 | 7.5 | 37 | 55 |
| 097-144 | 0.38 | 9.7 | 57 | 85 |

- Jacket Print & Cable Packaging

The length shipped shall be greater than or equal to than the ordered length. All cables shall have sequential length markings along the cable sheath, every two feet for cables marked in feet, or every meter for cables marked in meters. The cable length shall be no shorter than the marked length, but may be up to 1% greater than the marked length. If the initial cable marking (white characters) fails to meet the marking requirements, the cable must be remarked. The remarking is imprinted with yellow characters on a different portion of the cable sheath. Therefore, for any cable that contains two sets of cable marking, only the yellow marking should be used.

As required by the NESC, ANSI C2-1993, all cables manufactured after 1993 shall have a visual identifier resembling a telephone handset to identify them as telecommunications/data cables.

4.1		Cable End Fastening

To provide access for testing, the bottom end, “test tail”, shall be approximately three meters in length and easily assessable.  For wooden reels the end shall be protected by protection rings and shall be securely fastened to the outside of the reel flange by wood screws.  For plywood reels, the end shall be fed into the 2nd slot and be protected by wood lagging.

  - Information Accompanying the Reel

Each cable shall have a certified test data sheet attached to the reel in a waterproof wrapping containing the following information: cable number, date, customer name, ordered length, customer order number, ship length, customer cable code, customer reel number, customer attenuation specification(s), number of fibers, cable construction, fiber transmission data, bandwidth data (for multimode fibers only), and authorized signature.

  - Reel tag

Each cable shall have a reel tag fastened to the reel in a waterproof wrapping containing the following information: cable number, date, customer name, ordered length, customer order number, ship length, customer cable code, customer reel number, customer’s attenuation specification(s), number of fibers, beginning and ending sequential length markings, gross weight, net weight, Inspected by signature, remarks if applicable, date of manufacture by month and year.

  - Pre-Shipment End Sealing

The ends of all cables are sealed to prevent the entry of moisture during shipping, handling, storage, and installation.

- Reels

Reels shipping domestically will have lagging.  Each wooden reel shall be permanently marked with	the following information:

- Manufacturer

- Fiber Optic Cable

- An arrow and the wording “cable end” to indicate the position of the outside cable end

- An arrow and the wording “ROLL THIS WAY” to indicate the direction the reel should be rolled

- Reel Number

- “Made in the USA”.

The arbor hole in the flange shall have a nominal diameter of either 3 1/8” or 4 1/4” for wooden reels.

Rationale: Sections 4.0-5.0 provide instructions regarding cable printing and packaging to ensure that the cable is properly identifiable and does not suffer damage during shipment.

- Mechanical & Environmental Testing

Single-Mode Fibers: Per Telcordia GR-20, the magnitude of the attenuation change shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10% of test fibers. Cable aging allows for 0.10 dB/km average attenuation change with a magnitude of the maximum attenuation change for each individual fiber to be less than 0.25 dB/km. These attenuation values include a 0.05 dB allowance for measurement repeatability.

During mechanical and environmental testing evidence of cracking, splitting or other failure of the sheath components when examined under 5X magnification would result in failure of the proposed test requirements.  In addition, no fiber shall lose optical continuity because of the test.

Table 4 – Testing for Single Mode Fibers – Items below apply to loose tube cables unless otherwise stated for microcables.

Table 4 – Testing for Single Mode Fiber

| Cable Test | Test Method * | Requirement |
| --- | --- | --- |
| Tensile Loading and Bending | EIA/TIA-455-33 IEC 794-1-E1** | - Maximum diameter of mandrels and sheaves: 30 times the OD of the cable - Tensile load rating for long term operation: Variable depends on design - Tensile load rating for short term operation: Variable depends on design Requirement: The magnitude of the attenuation change shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10%. 90% < 0.05 dB Max. Added Loss 100% < 0.15 dB Max. Added Loss |
| Cyclic Flexing | TIA/EIA-455-104 IEC 794-1-E6** | - Bending Angle: 90° - Number of cycles: 25 - Diameter of mandrel: Varies with cable OD Requirement: The magnitude of the attenuation change shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10%. 90% < 0.05 dB Max. Added Loss 100% < 0.15 dB Max. Added Loss |
| Cyclic Impact | EIA/TIA-455-25 IEC 794-1-E4** | - 2 impacts at 3 locations with 4.4 N*m impact energy. Requirement: The magnitude of the attenuation change shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10%. 90% < 0.05 dB Max. Added Loss 100% < 0.15 dB Max. Added Loss |
| Compressive Loading | TIA/EIA-455-41 IEC 794-1-E3** | - Dimension of plates: 100 mm - Attenuation measured after removal of load. - Loose tube - Incidental load - 220 N/cm (125 lbf/in) for non-armored and armored cables for 1 minute or more - Long term load of 110 N/cm (63 lbf/in) for 10 minutes. - Microcables - Incidental load - 220 N/cm (125 lbf/in) for 1 minute or more Requirement: The magnitude of the attenuation change shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10%. 90% < 0.05 dB Max. Added Loss 100% < 0.15 dB Max. Added Loss |
| Twist | TIA/EIA-455-85 IEC 794-1-E7** | - Rotation Angle: 180° - Number of cycles: 10 Requirement: The magnitude of the attenuation change shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10%. 90% < 0.05 dB Max. Added Loss 100% < 0.15 dB Max. Added Loss |
| Low and High Temperature Bend | EIA/TIA-455-37 IEC 794-1-E11 | - Maximum mandrel diameter: 20 cable od - Temperature: -40°C for 4 hours and 70°C for four hours Requirement: The magnitude of the attenuation change shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10%. 90% < 0.05 dB Max. Added Loss 100% < 0.15 dB Max. Added Loss |
| External Freezing | EIA/TIA-455-98 IEC 794-1-F6*** | - Initial Temperature: -10°C (until frozen) - Temperature: -2°C for 24 hours - The temperature is allowed to rise to 23°C for 24 hours. Requirement: The magnitude of the attenuation change shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10%. < 0.05 dB Mean Added Loss < 0.15 dB Max. Added Loss |
| Fiber Strip ability | EIA/TIA-455-178 No equiv IEC proc | 5.0 N (1.1 lbf) on unaged and aged fiber 1.3N (0.3 lbf) on unaged and aged fiber |
| Temperature Cycling | EIA/TIA-455-3 IEC 794-1-F1** | •Temperature range: 23°C. –40°C & 70°C • Duration at each temperature: 24 hours Requirement: The average change in attenuation at the temperature extremes of –40°C and 70°C for the measured fibers shall not be greater than 0.05 dB\km. The magnitude of the maximum attenuation change of each individual fiber shall not be greater than 0.15 dB/km. 0.05 dB/km Mean Added Loss 0.15 dB/km Max Added Loss |
| Cable Aging | EIA/TIA-455-3 IEC 794-1-F1** | - Temperature: 85°C for 168 hours, -40° for 24 hours, 70°C for 24 hours, -40°C for 24 hours, 70°C for 24 hours, returned to 23°C. Requirement: The average change in attenuation from the original values observed before the start of the temperature cycling test shall not be greater than 0.10 dB/km. The magnitude of the maximum attenuation change for each individual fiber shall not be greater than 0.25 dB/km. 0.10 dB/km Mean Added Loss 0.25 dB/km Max Added Loss |
| Water Penetration | EIA/TIA-455-82 IEC 794-1-F5** | - Cable sample: 1 0.1 meter Requirement: Sample must show no signs of leaking with a one-meter static head of fluid applied for 24 hours. |

- Quality Control

7.1 Reel Testing

Each master reel shall be tested to ensure fiber integrity, attenuation, and cable length. Single mode fibers shall be tested at both 1310 and 1550 nm. Each master reel shall be given a unique identification and the test results documented. The manufacturer must maintain documentation such that the cable history may be traced to the individual fibers used in construction of the cable.

  - Certified Test Data

Each cable shall have a certified test data sheet attached to the reel in a waterproof wrapping containing the following information:  manufacturer, cable number, date, Customer Name Customer Name, ordered length, order number, ship length, cable code, reel number, attenuation specification(s), number of fibers, cable construction, fiber transmission data and authorized signature.

Rationale: All reels should be tested at both 1310 and 1550 nm to ensure fiber integrity.  All manufacturers must include appropriate documentation and full traceability throughout the manufacturing process to address appropriate remedies in case of any manufacturing-related problems.

- Fiber Specifications

This document provides fiber specifications for ITU-T G.652 D/ITU-T G.657 A1 single-mode fiber in cable constructions.  The requirements below must be met in cabled fiber form unless otherwise highlighted.  Dual rated fiber is important for bending-related attenuation performance in the field.

  - Standards References

The Optical Fiber shall meet or exceed the national and international standards for single-mode fiber described in the table below.

Table 5 – Optical Fiber Standards

| Standard | Designation | Description |
| --- | --- | --- |
| ITU-T | G.652D/G.657A1 | extended wavelength band |
| IEC 60793-2-50 | B1.3 | extended wavelength band |
| TIA-492-CAAB | Type IVa | dispersion-unshifted with reduced water peak |

The Optical Fiber shall be referenced in the national and international standards for applications utilizing single-mode fiber described in the table below.

Table 6 – Application Standards

| Standard | Description | Optical Fiber Reference(s) |
| --- | --- | --- |
| IEEE 802.3 | Ethernet for Subscriber Access Networks. (Includes both PON and point to point applications) | IEC 60793-2-50 B1.3 |
| ITU – G.983.3 | Broadband Passive Optical Network (BPON) | ITU-T G.652D |
| ITU – G.984.2 | Gigabit Capable Passive Optical Network (GPON) | ITU-T G.652D |

Rationale:  Fibers included in the cables must be compliant to the latest versions of internationally-recognized specifications to ensure appropriate compatibility and proper performance with various network protocols over the next several decades.  Fibers that meet both G.652D and G.657A1 provide additional bend insensitivity that can be helpful to minimize bending-related attenuation in the field

- Material

The Optical Fiber shall be comprised of 100% synthetic silica to ensure the best possible long-term mechanical and optical performance. Natural Quartz shall not be used in any proportion of the fiber core or cladding.  Customer Name reserves the right to require the manufacturer to provide objective evidence of the use of 100% synthetic silica for selected fibers.

The Optical Fiber in finished cable form shall meet the transmission specifications in all wavelength bands listed in the tables below.

Table 7. Attenuation

The fiber shall be classified as a “zero water peak” fiber, meeting the attenuation specifications below:

| Wavelength (nm) | Maximum Attenuation (dB/km) |
| --- | --- |
| 1310 | 0.35 |
| 1383 | 0.31 |
| 1490 | 0.27 |
| 1550 | 0.25 |
| 1625 | 0.27 |

Table 7.1 Attenuation vs. Wavelength Uniformity

| Range (nm) | Reference (nm)  |  |
| --- | --- | --- |
| 1285-1330 | 1310 | 0.03 |
| 1360-1480 | 1385 | 0.04 |
| 1525-1575 | 1550 | 0.02 |
| 1460-1625 | 1550 | 0.04 |

The attenuation in a given wavelength range does not exceed the attenuation of the reference wavelength () by more than the value . This data is in fiber form.

Table 7.2 Change in Attenuation at Water Peak

The optical fiber attenuation coefficient at the OH- absorption peak (1383 + 3nm) after exposure to hydrogen shall be <0.31 dB/km and <0.29 dB/km typically. This test simulates long-term hydrogen aging in installed cables. The test shall be performed in fiber form.

Table 7.3 Optical Fiber (not in a cable) Macrobending Attenuation

| Deployment Condition | Wavelength | Induced Attenuation |
| --- | --- | --- |
| 1 turn, 10 mm radius mandrel | 1550 nm | <0.075 dB |
|  | 1625 nm | <1.5 dB |
| 10 turns on a 15 mm radius mandrel | 1550 nm | <0.25 dB |
|  | 1625 nm | <1.0 dB |
| 100 turns, 30 mm radius mandrel | 1550 nm | <0.05 dB |
|  | 1625 nm | <0.05 dB |

Table 7.4 Point Discontinuities in Cable

No attenuation discontinuities greater than 0.10 dB at 1310 nm or 1550 nm shall be present in the finished cable

Table 8 Chromatic Dispersion

| Zero dispersion wavelength (o) | 1302-1322 nm |
| --- | --- |
| The maximum dispersion slope (So) at o | 0.090 ps/nm2-km |
| Typical dispersion slope | 0.087 ps/nm2-km |
| Dispersion Max at 1310 nm | 3.5 ps/nm2-km |
| Dispersion Max at 1550 nm | 18 ps/nm2-km |
| Dispersion Max at 1625 nm | 22 ps/nm2-km |

Table 9 Mode Field Diameter (MFD)

| at 1310 nm | 9.2+0.4 m |
| --- | --- |
| at 1550 nm | 10.4+0.5 m |

Note:  The 9.2 µm MFD is important for splicing performance in the field.

Table 10 Cable Cutoff Wavelength

| Cable cutoff wavelength (cc) | < 1260 nm |
| --- | --- |

Table 11 – Geometry

| # Glass Geometry |  |
| --- | --- |
| Cladding Diameter | 125.0 + 0.7 m |
| Core/Clad Concentricity Error | < 0.5 m, <0.2 m typically |
| Cladding Non-circularity | < 0.70% |
| Typical Splice Loss (profile alignment) | <0.02 dB |
| # Coating Geometry |  |
| Coating Diameter (colored) | 245 – 260 m |
| Coating/Cladding Concentricity Error | < 12 m |

Table 12 – PMD in Cable

| Fiber PMD Link Design Value (LDV)2 | < 0.1 pskm)1/2 |
| --- | --- |
| Maximum Individual Fiber | < 0.1 pskm)1/2 |
| Typical Fiber LMC PMD | < 0.02 pskm)1/2 |

Rationale for optical specifications: The highest quality fiber is manufactured from 100% synthetic silica derived from reactions of chemicals.  As a method of cost reducing products, manufacturers have been known to substitute natural quartz mined from the earth as the source of the silica.  Natural quartz can experience inferior mechanical and optical reliability versus synthetic silica, and the specification is designed to eliminate the possibility of issues related to that concern.   The number of wavelengths that are being used in optical systems is increasing rapidly, and it is expected that this trend will either continue or accelerate over the next decades as the network is used.  Low attenuation values across the entire optical spectrum, including the “water peak” band around 1383 nm are needed to ensure that the network will be compatible with all of the existing and future wavelengths expected to be deployed over the system in the next several decades.  The Chromatic Dispersion and fiber and coating geometry specifications are standard industry specifications designed to ensure compatibility and ease of splicing/connectivity.  PMD (Polarization Mode Dispersion) is an important parameter to specify for higher speed application requirements.

The Optical Fiber shall meet the Environmental specifications listed in Table 13 when tested in fiber form.

For all mechanical & environmental tests, the magnitude of the attenuation change at 1550nm shall be less than or equal to 0.05 dB for 90% of the test fibers and less than or equal to 0.15 dB for the remaining 10% of the test fibers.  These attenuation values include a 0.05 dB allowance for measurement repeatability.

Table 13 – Environmental Characteristics of optical fiber (performance in closures, pedestal and cabinets)

| # Operating Temperature | -60o C to +85o C |
| --- | --- |
| Temperature Dependence of Attenuation Induced attenuation at 1310, 1550, & 1625 nm at -60o C to +85o C | <0.05 dB/km FOTP 3 |
| Temperature – Humidity Cycling Induced attenuation at 1310, 1550, & 1625 nm at –10 - +85o C at 95% RH | <0.05 dB/km FOTP 72 |
| Water Immersion, 23oC Induced attenuation at 1310, 1550, & 1625 nm due to water immersion at 23 + 2o C | <0.05 dB/km FOTP 75 |
| Accelerated Aging (Temperature), 85o C Induced attenuation at 1310, 1550, & 1625 nm due to temperature aging at +85 + 2o C | <0.05 dB/km FOTP 67 |
| Full Spectrum Testing & Performance Assurance |  |

Table 14– Mechanical Characteristics (Customer Name reserves the right to request 3rd testing to verify mechanical performance)

| # Proof Test Level | 0.7 Gpa (100 kpsi) |
| --- | --- |
| # Dynamic Tensile Strength (aged and unaged) The median tensile strength of samples with a 0.5 meter gauge length is: | >3.8 Gpa(550 kpsi) |
| Dynamic Fatigue Parameter (nd). This test must be performed in accordance with FOTP-28, Measuring Dynamic Tensile Strength and Fatigue Parameters of Optical Fibers by Tension. | >20 |
| # Coating Strip Force (aged and unaged) The average force to mechanically strip the dual coating is: | >1.3 N (0.3 lbf.) and < 5.0 N (1.1 lbf.) |
| Coating Adhesion (Pull-out force, aged, 30 days, 85/85 and unaged) Tested in accordance with FOTP-105. | >6.0 N/cm |

The Optical Fiber shall meet the other specifications listed in Table 15.

Table 15 – Other Characteristics

| Effective Group Index of Refraction 1310 nm 1550 nm | 1.466 1.467 |
| --- | --- |
| Rayleigh Backscattering Coefficient (for 1ns pulse width) 1310 nm 1550 nm | -79.6 dB -82.1 dB |

Rationale: The fiber must be able to withstand all of the installation, operational and environmental situations expected over its lifetime with minimal changes in attenuation performance.  In addition, the fiber must be proof tested at 100 kpsi to ensure a minimal strength level.  The Dynamic Fatigue Parameter is important to ensure that the strength of the fiber does not degrade significantly enough to pose a mechanical problem with the cable.  If this number is low or highly variable, traditional cable design and manufacturing parameters designed to ensure multi-decade lifetime of the cable become suspect.   The coating strip force and adhesion specifications are important to ensure that the fiber is easily strippable, but is not easily delaminated from the glass to provide long-term protection of the glass.  The EIOR (Effective Group Index of Refraction) and Rayleigh Backscattering numbers are characterization number used in conjunction with optical test equipment.

Appendix D

MUTUAL NON-DISCLOSURE AGREEMENT

This Mutual Non-Disclosure Agreement is between ___________________(“7SPC”) and ____________________ (“Company”), a ________________ duly organized and existing under the laws of the State of _____________.   7SPC and Company wish to exchange certain proprietary and confidential information.  As used in this Agreement, a party who receives Confidential Information shall be referred to as the “Receiving Party” and a party who discloses Confidential Information shall be referred to as the “Disclosing Party.”

In consideration of the relationship of the parties, any access of the Receiving Party to Confidential Information of the Disclosing Party, and for other valuable consideration, the receipt and sufficiency of which is hereby acknowledged, the Receiving Party hereby agrees as follows:

- “Confidential Information” means all intellectual property, trade secrets, commercially valuable or proprietary information related to the Disclosing Party’s business and products, whether of a technical, business or other nature including, without limitation, information relating to the technology, customers, business plans, promotional and marketing activities, finances and other business affairs of the Disclosing Party whether or not it has been labeled as confidential, identified orally as being proprietary and/or confidential, or by the nature of the circumstances surrounding the disclosure of the information ought to be treated as confidential.  Confidential Information shall include all summaries, extracts, notes, analyses, compilations, interpretations and other documents prepared by the Receiving Party that contain, reflect or are based upon or generated from, in whole or in part, the Confidential Information.

- Each party agrees that all Confidential Information shall remain the secret and confidential property of the Disclosing Party and the Receiving Party shall not disclose any Confidential Information to any other party unless authorized by this Agreement.  The Receiving Party shall (i) hold the Disclosing Party’s Confidential Information in strict confidence and take reasonable precautions to protect such Confidential Information from unauthorized disclosure, (ii) take reasonable precautions to prevent Confidential Information from being disclosed to any person other than those directors, officers, employees, legal counselors, agents or consultants of the Receiving Party who need to know such Confidential Information and the Receiving Party shall be responsible to notify any such person of the existence of this Agreement and the Receiving Party shall be responsible for any breach of this Agreement by any of them, (iii) not make any use whatsoever of such Confidential Information except as may be required by the relationship with the Disclosing Party, (iv) not copy, decompile, disassemble or reverse engineer any such Confidential Information, and (v) not export or re-export (within the meaning of U.S. or other export control laws or regulations) any such Confidential Information.

- Without granting any right or license, the Disclosing Party agrees that the limitations on disclosure of Confidential Information by the Receiving Party shall not apply to information that the Receiving Party can document and prove that (i) such information is or becomes generally available to the public other than a result of a breach or improper action or inaction by the Receiving Party or any individual who has a need to know such Confidential Information, or (ii) was already in the Receiving Party’s possession or known prior to receipt from the Disclosing Party, or (iii) was independently developed without the use of any Confidential Information of the Disclosing Party and the Receiving Party did not have access to such Confidential Information. The Receiving Party may make disclosures if required by law or requested by any court with jurisdiction to disclose or produce any Confidential Information, the party so requested or required to disclose or produce any Confidential Information will (i) provide the other party with prompt notice thereof and copies, if possible, and, if not, a description, of the Confidential Information requested or required to be disclosed or produced so that the Disclosing Party may seek an appropriate protective order or waive compliance with the provisions of this Agreement.

- Upon expiration or termination of this Agreement or if a Disclosing Party so requests at any time, the Receiving party will return promptly to the Disclosing Party or promptly destroy all copies, extracts or other reproductions in whole or in part of all Confidential Information in the possession of the Receiving Party and the 
Receiving Party will destroy or cause to be destroyed all copies of any memoranda, notes, analyses, compilations, studies or other documents prepared by the Receiving Party or for its use based on, containing or otherwise reflecting any Confidential Information of the Disclosing Party; provided, that each party shall be entitled to retain any electronic copies backed up in the ordinary course of business. Such destruction will, if requested, be certified in writing to the furnishing party by an authorized officer supervising such destruction. Notwithstanding the return or destruction of the Confidential Information, the parties will continue to be bound by their obligations under this Agreement.

- The Receiving Party understands that nothing herein (i) requires the disclosure of any Confidential Information of the Disclosing Party or (ii) requires either party to perform any work or to enter into any transaction, relationship, or agreement. Neither party will share or disclose any of the work it does with the other party or the nature of its relationship for any reason without the express prior written permission of the other party.

- The Disclosing Party provides Confidential Information without warranties of any kind and is not liable for any damages arising out of the Receiving Party’s use of Confidential Information.  Neither this Agreement nor any disclosure of Confidential Information made under it grants the Receiving Party any right, title, interest or license to the Confidential Information or to any trademark, copyright patent or any other intellectual property right now or subsequently owned or controlled by Disclosing Party. The exchange of Confidential Information under this Agreement does not create any joint or fiduciary relationship, or authorize either party to act or speak on behalf of the other.

- The Receiving Party acknowledges and agrees that due to the unique nature of the Disclosing Party’s Confidential Information, there may be no adequate remedy at law for any breach of its obligations hereunder, which breach may result in irreparable harm to the Disclosing Party, and therefore, that upon any such breach or any threat thereof, the Disclosing Party shall be entitled to seek appropriate equitable relief, without the requirement of posting a bond, in addition to other remedies available at law.

- In the event that any of the provisions of this Agreement shall be held by a court or other tribunal of competent jurisdiction to be illegal, invalid or unenforceable, the Parties agree to renegotiate such provision in good faith. In the event that the Parties cannot reach a mutually agreeable and enforceable replacement, such provisions shall be limited or eliminated to the minimum extent necessary so that this Agreement shall otherwise remain in full force and effect.

- This Agreement shall be governed by the laws of the State of Tennessee without regard to its conflicts of law provisions.

- The prevailing party in any action to enforce this Agreement shall be entitled to costs and reasonable attorneys’ fees. No waiver or modification of this Agreement will be binding upon a party unless made in writing and signed by a duly authorized representative of such party and no failure or delay in enforcing any right will be deemed a waiver.

- This Agreement supersedes all prior discussions and writings and constitutes the entire agreement between the parties with respect to the subject matter hereof.

- Each of the parties to this Agreement represents and warrants that it has carefully read this Agreement, that it fully understands its final and binding effect, that it has had an opportunity to consult with an attorney of its own choosing prior to signing and that the person signing this Agreement has the power and authority to execute and deliver the Agreement on behalf of the party whom s/he represents.

- This Agreement comprises the complete and integrated agreement of the parties related to confidentiality and supersedes and terminates all prior agreements, leases, promises, understandings, and representations, written or oral, on such subject matter. In the event of any conflict between the provisions of this Agreement and those of any other agreements, the provisions of this Agreement shall control.

- The term of this Agreement is for three (3) years from the Effective Date. Either party may terminate this Agreement at any time by giving the other party thirty (30) days written notice. The parties’ obligations hereunder with respect to Confidential Information shall survive the termination or expiration of this Agreement and shall continue in full force and effect for a period of two (2) years from the termination or expiration date.

IN WITNESS WHEREOF the parties have duly executed this Agreement effective as of the date the last party signed this Agreement (“Effective Date”).

Company						7SPC

Signature:						Signature:

Name:							Name: _____________________________________

Title:							Title: _____________________________________

Date: 							Date:

Address for Notice:					Address for Notice:

_____________________________				_____________________________

_____________________________				_____________________________

_____________________________				_____________________________