SAFETY DATA SHEET

SECTION 1 - IDENTIFICATION OF THE MATERIAL AND SUPPLIER

- Product name:			ULTIMATE

- Product Code:			None allocated

- Other names:			None allocated

- Recommended use:		CLEANER/SANITISER

- Supplier name:			Blandeen Pty Ltd

- Supplier address:			Po Box 274 Kilcoy QLD 4515

- Supplier telephone number:		0411 884 942

- Emergency number:		poisons Information Centre 131126 or Distributor 0755314890

- Distributor name:			DUNCKERS

- Distributor address:		3A Whalley Creek Close Nambour QLD 4560

- Distributor telephone number:	07 54413390

- Distributor fax number:		07 54412630

SECTION 2 - HAZARD IDENTIFICATION

Hazard Classification:	Hazardous according to the criteria of the Globally Harmonised System of Classification and

Labelling of Chemicals (GHS).

Not Classified as Dangerous goods according to the criteria of the Australian Code for the

Transport of Dangerous Goods by Road and Rail.

Hazard Categories:	Eye Damage Irritation – Category 2A

Skin Corrosion/Irritation – Category 2

Signal Word:		Warning

Pictograms:

Hazard Statements:	H315	Causes skin irritation.

H319	Causes serious eye irritation.

Precautionary Statements:

Prevention	P264	Wash contaminated skin thoroughly after handling.

P280	Wear protective gloves/ protective clothing/eye protection/ face protection.

Response	P302+P352		If on SKIN: Wash with plenty of soap and water.

P332+P337+P313	If skin/eye irritation occurs: Get medical advice/attention.

P305+P351+P338	If in EYES: Rinse cautiously with water for several minutes. Remove contact 					lenses, if present and easy to do. Continue rinsing.

P362			Take off contaminated clothing and wash before re-use.

Disposal		P501			Dispose of contents/container to an approved waste disposal plant.

SECTION 3 - COMPOSITION / INFORMATION ON INGREDIENTS

| Ingredients | CAS Number | Proportion |
| --- | --- | --- |
| QUATERNARY AMMONIUM COMPOUND | 68424-85-1 | <10% |
| Ingredients determined to be non-hazardous | Various | To 100% |

SECTION 4 - FIRST AID MEASURES

If poisoning occurs contact a Doctor immediately or the Poisons Information Centre on 131126.

- First Aid Facilities:			Normal washroom facilities / Eye wash station / Safety shower

- Eye Contact:			Rinse eyes immediately (including under eye lids) for at least 20 minutes with

running water while holding eyelids open. Seek medical advice if symptoms persist.

- Skin Contact:			Remove all contaminated clothing and footwear. Wash the skin with plenty of

water and mild detergent for at least 20 minutes. Seek medical advice if symptoms persist.  Fully launder clothes before re-use.

- Ingestion:				DO NOT INDUCE VOMITING. Seek medical advice if feel unwell. Rinse mouth with

water. Give water to drink. If vomiting occurs, give further water to dilute.

- Inhalation:			Remove to fresh air. Seek medical advice if symptoms are severe.

- Advice to Doctor:			Treat symptomatically. All treatments should be based on observation and

symptoms of distress.

SECTION 5 - FIRE FIGHTING MEASURES

- Extinguishing Media:		Not flammable. Foam, dry chemical or fine water spray.

- Hazards from Combustion

Products: 			Water based. Not combustible.

- Fire Fighters:			Fire fighters to wear full protective clothing and self-contained breathing apparatus. 					Move upwind of fire. Keep containers that are exposed to heat cool with water spray.

- Hazchem Code:			None.

- Flashpoint:			No data available.

SECTION 6 - ACCIDENTAL RELEASE MEASURES

- Emergency Procedures:		None.

- Methods and Materials for

Containment and Clean Up:	Minor spills normally do not require any special clean up measures. In the event of a major spill prevent spillage from entering drains, water courses, sewers or any body of water. Wear protective equipment (as per section 8) to prevent skin and eye contamination.  Contain and absorb any spillage with sand, earth or inert material. Scoop into a sealable container for disposal by an approved agent according to local conditions. Any residual deposits may be slippery. Wash down all areas with water. If contamination of sewers or waterways has occurred notify your local emergency services.

SECTION 7 HANDLING AND STORAGE

- Handling:				Keep out of reach of children. When handling do not eat, drink or smoke.

Avoid contact with skin and eyes. Use in a well-ventilated area. Wash hands with water after handling.

- Storage:				Store in a cool, well ventilated, dry place away from direct sunlight. Keep

container closed at all times.

- Incompatibility:			None.

SECTION 8 EXPOSURE CONTROLS / PERSONAL PROTECTION

- Exposure standards:		There are no exposure standards available for this product.

- Biological limit values:		None established for this product.

- Engineering controls:		Ensure ventilation is adequate and avoid generating mists of the product. Ensure

eyewash and/or a safety shower in working area. Keep container closed when not in use.

- Personal Protective Equipment:

Eye protection:	Avoid contact with eyes. The use of safety goggles/ glasses with side protection is recommended.

Skin Protection:	Wear chemical resistant gloves, overalls, apron and appropriate footwear.

Respiratory:	If ventilation is inadequate wear a respirator to protect against exposure to high vapour concentrations. Respirators should comply with AS/NZS 1715 and AS/NZS 1716.

Other:	Do not eat, drink or smoke whilst handling product. Wash hands after use.

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES

- Appearance:		A clear yellow liquid.

- Odour:			Lemongrass odour.

- pH:			6 - 7 (Neat).

- Vapour Pressure:		Not available.

- Vapour Density:		Not available.

- Boiling Point:		100 °C.

- Freezing/Melting Point:	Not available.

- Solubility:			Soluble in water.

- Specific Gravity:		1.0

- Flash Point:		Not flammable.

- Flammability limits:	Not available.

SECTION 10 - STABILITY AND REACTIVITY

- Chemical stability:		Stable at normal temperatures and pressure.

- Conditions to avoid:	None

- Incompatible materials:	None.

- Hazardous decomposition

products:			None.

- Hazardous reactions:	None.

SECTION 11 - TOXICOLOGICAL INFORMATION

- Ingestion:			May cause irritation or allergic reactions in sensitive individuals.

- Skin Contact:		May cause irritation or allergic reactions in sensitive individuals.

- Eye Contact:		Causes serious eye irritation – will cause redness and watery eyes.

- Inhalation:		None.

SECTION 12 - ECOLOGICAL INFORMATION

- Ecotoxicity:			None available.

- Persistence and degradability:	None available.

- Bioaccumulative potential:		None available.

- Mobility:				None available

This product is miscible in water. Do not discharge bulk quantities into drains, waterways, sewer or environment. Notify your local authority if this occurs.

SECTION 13 - DISPOSAL CONSIDERATIONS

- Disposal:			To dispose of quantities of undiluted product refer to your State Land Waste

Management Authority. Do not discharge product into drains or waterways. Dispose of material through a licensed accredited waste disposal contractor.

SECTION 14 - TRANSPORT INFORMATION

- UN Number:			None.

- UN Proper Shipping Name:		None.

- Class and Subsidiary Risk:		None.

- Packing Group:			None.

- Hazchem Code:			None.

- Special precautions: 		None.

SECTION 15 - REGULATORY INFORMATION

- SUSMP Classification:		None.

- Regulatory Information:		Hazardous according to the criteria of the Globally Harmonised System of 						Classification and Labelling of Chemicals (GHS).

SECTION 16 - OTHER INFORMATION

- Issue Date:		November 2016

- Version:			1.0

- Revision Date:		24th March 2020

This SDS summarises at the date of issue our best knowledge of the health and safety hazard information of the product and how to safely handle and use the product in the workplace. Since the supplier and/or distributor cannot anticipate or control the conditions under which the product may be used, every user must prior to usage review this SDS in the context of how the user intends to handle and use the product in the workplace. If clarification or further information is required to ensure that an appropriate risk assessment can be made, the user should contact this company.

END OF SDS